Advanced Signal Processing

Exam questions

Abstract

The oral exam lasts about 15 minutes per student, with 5 additional minutes for grading. Students randomly draw two topics from two different course areas (A: 1-4, B: 5-8, or C: 9-12) at the start of the exam and choose the order of discussion. They present their understanding of each topic, supported by a sheet of notes and using the blackboard for visuals, while examiners ask questions. The exam is graded on the 7-step scale and conducted in English or Danish by the course lecturers.

Contents

1. Wiener Filtering	2

2. Least-Mean-Squares Adaptive Filtering	3

3. RLS Adaptive Filtering	4

4. Multidimensional Fourier Transform	5

5. Signal Spaces	6

6. Detection theory	7

7. Minimum Mean Squared Error (MMSE) and Linear MMSE estimation	8

8. Kalman Filtering (scalar and vector versions)	9

9. One-step linear predictor	10

10. The Levinson-Durbin algorithm	11

11. AR-filter design by modelling	12

12. The Lattice FIR filter	13

1. Wiener Filtering

Wiener filtering is a powerful linear technique used to minimize the mean squared error (MSE) between a desired signal d(n) and the filter output y(n). It is ideal for stationary signals, where signal statistics remain constant over time. The Wiener filter operates by finding an optimal set of weights w that minimizes the error e(n) between d(n) and y(n).

ex. 1 FIR filter with M weights i=0M-1woiri-k=p-k

ex. 1 FIR filter with M weights i=0M-1woiri-k=p-k

1. The filter output is computed as:

yn=k=0∞wk*un-k

where u(n) represents the input signal and wk​ are the filter weights.

Steepest descent

- Assumes knowledge of signal statistics (R,p).

- Optimal convergence to Wiener Solution as 𝑛 → ∞

- On Wiener-filtering (update rule):

w(n + 1) = w(n) + μ[p - Rw(n)]

Steepest descent

- Assumes knowledge of signal statistics (R,p).

- Optimal convergence to Wiener Solution as 𝑛 → ∞

- On Wiener-filtering (update rule):

w(n + 1) = w(n) + μ[p - Rw(n)]

The error signal is defined as:

en=dn- yn

2. The optimization goal is to minimize the MSE:

J= Een2

where R=E[uuH] is the autocorrelation matrix of the input signal

p=E[ud*] is the cross-correlation vector between the input u(n) and the desired signal d(n)

wo=wo0,wo1,…,wo,m-1T (Optimal weights)

where R=E[uuH] is the autocorrelation matrix of the input signal

p=E[ud*] is the cross-correlation vector between the input u(n) and the desired signal d(n)

wo=wo0,wo1,…,wo,m-1T (Optimal weights)

3. To achieve this, the Wiener-Hopf equations are used:

Rwo=p,       wo=R-1p      (matrix form)

The principle of orthogonality ensures that, at optimality, the error e(n) is orthogonal to all components of the input u(n). This property guarantees that the filter minimizes the MSE.

While the Wiener filter is effective, its implementation requires knowledge of R and p, which must be estimated in practice. Additionally, computing R-1 can be computationally expensive for high-dimensional systems.

Applications: Wiener filters are widely used in noise cancellation, signal prediction, and system modeling.

Extra: 
1. Wiener optimal solution

∇k J= ∇kEene*n= -2Eun - ke*n
∇k J= Eun - keo* n= 0

Principle of orthogonality: when the filter operates optimally (i.e., J is minimum), eon is orthogonal to each input sample un – k contributing to the estimation of the desired response.

2. Wiener-Hopf equations

From the Wiener optimal solution, we can expand by inserting eo* :

E un - kd*n- i=0∞woiu*n - i=0

i=0∞woiEun - ku*n - k=Eun - kd*n

This leads to the Wiener-Hopf equations

i=0∞woiri-k=p-k,       ex 1 ↑

With:           Eun – ku*n – i= ri – k

Eun - kd*n= p-k

2. Least-Mean-Squares Adaptive Filtering

LMS adaptive filtering is a method to minimize the mean squared error between a desired signal and the filter output. It adjusts filter coefficients iteratively using stochastic gradient descent (SGD), making it simple and computationally efficient for real-time applications.

dn: Desired signal

en: Error between the desired sig. and the filter output.

dn: Desired signal

en: Error between the desired sig. and the filter output.

1. The goal of LMS filtering is to minimize the MSE cost function:

J= Een2=Edn-wHnun2

2. Approximation Using Instantaneous Error:

Instead of using the expectation operator, LMS minimizes the instantaneous squared error:

Jsn= en2=ene*n

3. Gradient Computation:

The gradient of the cost function Jsn with respect to the filter weights wn is:

∇Jsn=∂Jsn∂wk*n=-2un - ke*n

4. Weight Update (Minimization Step):

To minimize Jsn, the filter weights are updated in the direction of the negative gradient:

wkn + 1= wkn-12μ∇Js,kn

Substituting ∇Js,kn, the update rule becomes:
wkn + 1=wkn+μ⋅un - ke*n

5. Stability Criterion:

As for steepest descent, convergence/stability of LMS is guaranteed when: 0<μ<2λmax. Where λmax is the largest eigenvalue of the input signal’s autocorrelation matrix. In practice, μ has to be sufficiently small for a robust behavior NOTICE: Instantaneous estimates for 𝐑 and 𝐩 are included by the feedback and averaged over time.

Normalized LMS: LMS can suffer from unstable updates when the input signal has high power. NLMS resolves this by normalizing the step size with the signal power i.e. un2: ¨

w n + 1= w n+μδ+un2⋅un - ke*n

Where the 𝛿 is included for numerical stability. NLMS reduces the risk of large updates and ensures faster convergence.

Exponential Window

Exponential Window

3. RLS Adaptive Filtering

Sliding Window

Sliding Window

Growing Window

Growing Window

The Recursive Least-Squares (RLS) algorithm minimizes the weighted least squares error between a desired signal d(n) and the filter output y(n). It uses past data with exponentially decreasing weights, enabling fast convergence and adaptability to non-stationary signals.

i1 and i2 : segment’s length
w: filter weights.

i1 and i2 : segment’s length
w: filter weights.

1. Minimize Cost Function: The RLS algorithm minimizes a least-squares cost function J2w, which sums the squared errors over a data segment:

J2w=i=i1i2e2i=i=i1i2di-yi2

λ: forgetting factor. Assigning less weight to older data ↓

λ: forgetting factor. Assigning less weight to older data ↓

2. Weighted Cost Function: To prioritize recent data, the RLS algorithm uses a weighted cost function with a weighting factor (βn,i):

Jβwn=i=1nβn,ie2i
=i=1nβn,idi-wTnui2

3. Exponential Weighting: The weighting function (βn,i) can take the form of the exponential function:

βn,i=λn-i, 0<λ≤1, i=1,2,…,n

4. Regularization Term: To prevent noise from affecting early updates, a regularization term δ is added to the cost function:

Jβwn=i=1nλn-ie2i+δλn|wn|2

5. Gradient: The gradient of this cost function with respect to the filter weights is calculated and set to zero, resulting in the following system of equations:

∇Jβwn=0
i=1nλn-iuiuTiwn+δλnwn=i=1nλn-iuidi
Φnwn=zn

Φn is an M × M time-average auto-correlation matrix of the filter input. zn is an M × 1 time−average cross−correlation vector.

To solve the weighted normal equations:

Φnwn=zn

One must solve:

wn=Φ-1nzn

6. Correlation Matrix: Using exponential weight, the recursive computation of the correlation matrix Φn is:

Φn=λΦn-1+unuTn

This equation states that the present value of the correlation matrix Φ[n] is computed by the old matrix Φ[n − 1] and the outer product of the newly arrived input sequence. The same approach for recursive computation of the vector z[n] leads to:

zn= λzn-1+undn

8. Matrix Inversion: After calculating Φ[n] and z[n], the filter weights are obtained by inverting the matrix Φ[n] and solving the system. However, this can be computationally expensive, especially for large filter lengths. To address this, the Matrix Inversion Lemma is used giving us:

Φ-1n=λ-1Φ-1n-1-λ-1knuTnΦ-1n-1

9. Weight Update: The filter weight update is given by: wn=Φ-1nzn=wn-1+knξn

kn: Gain vector, determines the weight adjustment
ξn: Priori estimation error

4. Multidimensional Fourier Transform

The multidimensional Fourier transform generalizes the Fourier transform to signals defined in multiple dimensions. It is a critical tool for analyzing spatial and temporal signals in fields like imaging, radar, telescopes and medical diagnostics.

1. Fourier Series

Assume a function f  defined over a finite interval T: 
ft=k=-∞∞akei2πktT,      ak=1TTfte-i2πktT dt

Fourier Transform (Analysis): Given a function f∈L1R, the Fourier transform F is defined as:
Fω=F{ft}=Rfte-i2πkt dt

Inverse Fourier Transform (Synthesis): Given a function (F∈L1R), the inverse Fourier transform (F-1) is defined as:
ft=F-1{Fk}=RFkei2πkt dk

Any periodic function can be represented exactly using Fourier series.

General Properties of Fourier Transform
Spectrum: Sfk=Fk2, Fk=F{ft}
Bandwidth: Fk=0   ∀k∉B

Nyquist Sampling Theorem: Let f be a signal with band-limit B. Then, we can reconstruct f exactly as long as the sampling frequency is greater than 2B.

Duality Property: The Fourier transforms are related by the following relations:
F{f}=F-1{f-}F{F{f}}=f-,  where f-x=f-x

| Theorem | fx | Fk |
| --- | --- | --- |
| Similarity | f(ax) | 1aFka |
| Addition | fx+g(x) | Fk+G(k) |
| Homogeneity | af(x) | aF(k) |
| Time/phase shift | f(x-a) | ei2πakF(k) |
| Convolution | fx*g(x) | Fx*G(x) |
| Derivative | f'(x) | i2πk⋅F(k) |

Parity of a Function:
fx is even if f-x=fx.
fx is odd  if f-x=-fx.

Fourier Transform follows the properties:

s

Rayleigh’s Theorem (Energy):          Parseval’s Theorem (Power):
Rnfx2dx=RnFω2 dω          Rnfxg*(x)dx=FωG*ωdω

Rayleigh’s Theorem (Energy):          Parseval’s Theorem (Power):
Rnfx2dx=RnFω2 dω          Rnfxg*(x)dx=FωG*ωdω

2. Multidimensional Fourier Transform

For a signal fx with x=x1,x2,…,xNT∈Rn.

The Fourier transform is:
Fω=F{fx} =Rnfxe-i2πxTω dx

The inverse Fourier transform reconstructs the signal: 
fx=RnFωei2πxTω dω

Note that the exponential function (ei2πxTω) is determined by the product of the vectors 𝐱 and ω where ω=ω1,…,ωnTis the multidimensional frequency vector: x⋅ω=x1ω1+…+xnωn

Why have a vector of frequencies? For a fixed ω, the exponential function represents a family of parallel lines in the x1,x2-plane (or in the spatial domain), normal to ω, with spacing: distance=1ω12+ω22= 1ω Periodicity: We may now parametrize x using the normalized frequency vector. Give the starting point a= a1, a2T we obtain that: xt=a+tωω

Therefore, the complex exponential function adopts the following form: e±i2πxTtω=e±i2πaTω⋅e±i2πtω

The first term is constant, not depending on t. The second term is periodic with period 1/||ω||. Therefore, although the frequency now is a vector ω we can keep most of the terminology for the one-dimensional case.

| Theorem | fx | Fω |
| --- | --- | --- |
| Similarity | fax with a∈Rn | 1a1a2⋯anFω1a, ω1a,⋯,ωna |
| Addition | fx+g(x) | Fω+G(ω) |
| Shift | f(x-a) | ei2πaωF(ω) |
| Conv. | fx*g(x) | FωG(ω) |
| Deriv. | ∂∂xif(x) | i2πωi⋅F(ω) |

Separability: If a multivariable function can be written as a product of single-value:
fx1, x2, … , xn=f1x1f2x2⋯fnxn 
Ffx1, x2, … , xn=Ff1x1Ff2x2⋯Ffnxn

General Stretch Theorem: For any linear transformation, A, of the input data, the Fourier transform is: F{fAx}=1det(A) F(A-Tω) .

General Stretch Theorem: For any linear transformation, A, of the input data, the Fourier transform is: F{fAx}=1det(A) F(A-Tω) .

Special case – Rotations: The rotation matrix, R, has the property that R=RT and det(R) = 1, i.e., A rotation matrix R in the spatial domain results in the same rotation in the frequency domain: F{fRx}=FRω.

Special case – Rotations: The rotation matrix, R, has the property that R=RT and det(R) = 1, i.e., A rotation matrix R in the spatial domain results in the same rotation in the frequency domain: F{fRx}=FRω.

5. Signal Spaces

A signal space is an inner product space where all elements have finite norm. Each element is called a signal.

Examples of signal spaces:

Finite length real-values discrete time signals:
x=x1,x2…,xNT,y=y1,y2,…,yNT

Inner Product Space: x, y=n=1Nxnyn=xTy=yTx    matrix form
Energy of x: x2=x,x=nxn2

If x and y are complex-valued:

x, y=n=1Nxnyn*=xHy   (matrix form)

x2=nxn2

where xH is the Hermitian transpose of x (i.e., transpose and complex conjugate *).

Inner Product Space: A signal space is often equipped with an inner product to measure similarity or orthogonality between signals.

Norm: The norm (magnitude) of a signal is given by x=x,x.

Orthogonality: Two signals x and y are orthogonal if: x,y=0. Orthogonal signals carry independent information, which is key in applications like multiplexing.

Projection of signal x onto signal y: x,y=x, yy2y , if y2=1, then x,y=x, yy

Applications and Examples

- Fourier Analysis:

  - Sinusoidal basis functions ϕk(t)=ei2πkt decompose signals into their frequency components.

- Orthogonal Projections:

  - Used in least squares for minimizing the error between a signal and its approximation.

- Signal Approximation:

  - Signals can be approximated by projecting onto a subspace spanned by orthogonal basis functions.

6. Detection theory

How do we make a decision?

- Daily life  no specific ruleset,

- Formulate decision rule

- Optimality criterion

- Formulate decision rule

- Optimality criterion

Sometimes a particular rule or method

Engineers:

The goal of detection theory is to select one of 𝐾 mutually exclusive hypotheses in the set H = h0, . . , hk-1, based on some observations, x = x1, … , xnᵀ. This data vector is deterministic but, we model it as a realization of a random vector, denoted with pdf fX.

Probability Model for Decision Problems: To do this, a decision rule is used, which is a function Hx, that assigns a hypothesis for each possible observation i.e.

H : rangeX→H.

We mark the output of a decision rule with a hat to indicate that the decision Hx is not necessarily equal to the true hypothesis H. We may have some prior knowledge about the likeliness of a hypothesis being true: Pk is the prior probability of the hypothesis. The prior probabilities sum to one: P0+…PK+1=1. After making the observations, our view of the probability of a given hypothesis being true changes. Now we say

P''H is true given x''=PH=hkx,

Also known as the a posteriori probability of hk being true. Where: fX|Hxhk is the likelihood for hk and is a conditional pdf of X.X depends on which of the hypotheses are true, thus it's the conditional pdf of X under hypothesis hk (likelihood for hk) , fX|H(x|hk).

Bayes' theorem can expand the a posteriori probability and include the prior and likelihood:

PH=hkx= fX|H(x|hk)PkfXx

The denominator will be disregarded since it’s the same regardless of hk and will not enter the decision rules when derived. If we consider the decision rule evaluated at the random data, X, then the decision HX is a random variable.

Maximum A Posteriori Probability (MAP) Decision Rule: The MAP decision rule picks the hypothesis which has the highest probability of being correct. That is:

HMAPx=argmaxhk∈HfX|HxhkPk

The prior probability affects the probability significantly. Pk=0 means the hypothesis will never be selected, and the opposite if Pk=1. Furthermore, the more probable a hypothesis is (before we observe the data), the more convincing the data needs to be in order for us to change our minds.

Decision Regions: The regions of the x-axis for which the rule will select a respective hypothesis.

Maximum Likelihood (ML) Decision Rule: In cases where no prior probability is available or undesirable to use, one can instead set all prior probabilities equal. This results in the ML-decision rule:

HMLx=argmaxhk∈HfX|Hxhk

Bayes' Decision Rule: Instead of maximizing a probability, Bayes' decision rule minimizes a cost/risk of choosing a particular hypothesis. For this, a cost Ckk is assigned for if hk is decided and hk is true. Diagonal cost should be low, since these correspond to “true” decisions.

|  | H=h0 | H=h1 | ⋯ |
| --- | --- | --- | --- |
| H=h0 H=h1 ⋮ | C00 C10 | C01 C11 |  |

The expected cost/risk for decision hk given the data is obtained as the sum of the costs weighted by the conditional probability for hypothesis H given x:

Chkx = k'=0K-1Ckk' PH=hk'x.

Bayes' decision rule then chooses the hypothesis with the lowest expected cost:

HBayesx=argminhk∈HChkx.

Thresholds for the Binary Case

The binary decision rule can be written as:

H=h0,  &if Λx≥γh1,  &otherwise,  Λx=fX|HxH0fX|HxH1

where a likelihood ratio denoted by Λx is compared to a threshold γ which does not depend on x.

The threshold γ is 1 for ML, P1P0 for MAP, and for Bayes: γBayes=C11-C01P1C00-C10P0.

Other Decision Rules MAP, ML, and Bayes are not the only decision rules. Two prominent examples of other decision rules are: 1. The Neyman-Pearson rule, 2.The Mini-max rule

7. Minimum Mean Squared Error (MMSE) and Linear MMSE estimation

8. Kalman Filtering (scalar and vector versions)

← Block Diagram of the scalar Kalman Filter:

← Block Diagram of the scalar Kalman Filter:

System and Channel Model for the scalar Kalman Filter

System and Channel Model for the scalar Kalman Filter

Vector version
When extending to a vector version, the models become:
y(n) = H(n)y(n - 1) + z(n), n = 1,2, …
x(n) = A(n)y(n) + w(n), n = 1,2, …

𝑯(𝑛) and 𝑨(𝑛) are now known matrices describing the relation between previous estimates and new. 𝒛(𝑛) and 𝒘(𝑛) is now vectors of noise. The steps of the Kalman filter changes to:
Prediction Step:
y(n|n - 1) = H(n)y(n - 1|n - 1)
x(n|n - 1) = A(n)y(n|n - 1)
R(n|n - 1) = HnRn - 1n - 1HnT + Qz(n)

Updating Step:
y(n|n) = y(n|n - 1) + B(n)(x(n) - x(n|n - 1))
R(n|n) = (I - B(n)A(n))R(n|n - 1)
B(n)     = Rnn - 1AnT(AnRnn - 1AnT+ Qw(n))-1

Initialization:
y0     = μy(0)
R(0|0) = cov(y(0))

Vector version
When extending to a vector version, the models become:
y(n) = H(n)y(n - 1) + z(n), n = 1,2, …
x(n) = A(n)y(n) + w(n), n = 1,2, …

𝑯(𝑛) and 𝑨(𝑛) are now known matrices describing the relation between previous estimates and new. 𝒛(𝑛) and 𝒘(𝑛) is now vectors of noise. The steps of the Kalman filter changes to:
Prediction Step:
y(n|n - 1) = H(n)y(n - 1|n - 1)
x(n|n - 1) = A(n)y(n|n - 1)
R(n|n - 1) = HnRn - 1n - 1HnT + Qz(n)

Updating Step:
y(n|n) = y(n|n - 1) + B(n)(x(n) - x(n|n - 1))
R(n|n) = (I - B(n)A(n))R(n|n - 1)
B(n)     = Rnn - 1AnT(AnRnn - 1AnT+ Qw(n))-1

Initialization:
y0     = μy(0)
R(0|0) = cov(y(0))

Application: 
sensor fusion

Application: 
sensor fusion

↑ Block Diagram of the vector Kalman Filter

↑ Block Diagram of the vector Kalman Filter

↑ System and Channel Model for the vector Kalman Filter

↑ System and Channel Model for the vector Kalman Filter

9. One-step linear predictor

The one-step linear predictor is a method for predicting the next value in a time series based on the linear combination of previous values. It is commonly used in signal processing, such as speech analysis or AR (autoregressive) modeling.

Concept

The idea is to predict x[n], the current sample, using a weighted sum of its p past samples:

xn=-k=1pak⋅xn-k

xn: the predicted value.

xn-k: the previous samples (past observations).

ak​: the predictor coefficients, which determine how much weight to assign to each previous sample.

The error between the actual and predicted value is the prediction error:

en=xn-xn

Steps to Derive the One-Step Linear Predictor

1. Model the Signal as an AR Process:
Assume the signal x[n] is generated as:

xn=-k=1pak⋅xn-k+ en

ak​: predictor coefficients to be determined.

en: prediction error (noise).

2. Formulate the Prediction:
Use the past p samples to predict the current sample:

xn=-k=1pak⋅xn-k

The goal is to find ak​ that minimizes the error en=xn-xn

3. Minimize the Mean Squared Error (MSE):
Apply the principle of least squares:

Minimize Een2=

Exn+ k=1pak⋅xn-k2

This ensures the best-fit predictor coefficients.

4. Solve for Coefficients ak:

- Use the Yule-Walker equations to relate ak​ to the autocorrelation rm of xn.

- Efficiently compute ak​ using the Levinson-Durbin algorithm, avoiding matrix inversion.

5. Predict the Signal:
Substitute ak and past samples into the predictor formula:

xn=-k=1pak⋅xn-k

The result is the one-step prediction of xn.

Applications

- Speech Processing: Predictive coding for compression (e.g., LPC in audio codecs).

- Error Prediction: Useful in noise reduction and signal denoising.

- AR Models: Fundamental for spectral estimation and time-series analysis.

The one-step linear predictor is an essential tool in AR-based systems, offering both predictive capability and efficient error minimization.

10. The Levinson-Durbin algorithm

The Levinson-Durbin algorithm is an efficient method for solving the Yule-Walker equations used in linear prediction analysis, especially in autoregressive (AR) modeling. Instead of performing expensive matrix inversion, it recursively computes the predictor coefficients for increasing model orders.

Key Steps of the Levinson-Durbin Algorithm

1. Initialization:

- Start with the autocorrelation rm of the signal.

- Set the initial prediction error: E0=r0

2. Iterative Computation for Each Order m (from 1 to p):

E0=r0                                            Initial prediction error

for m=1 to p

km=-rm+k=1m-1am-1,k⋅rm-kEm-1     Reflection Coefficient
     am,m=km                                      Update Predictor Coefficients

for k=1 to m-1

am,k=am-1,k+km⋅am-1,m-k  Update Predictor Coefficients

end

Em=Em-1⋅(1-km2) 	           Update the Prediction Error

end

E0=r0                                            Initial prediction error

for m=1 to p

km=-rm+k=1m-1am-1,k⋅rm-kEm-1     Reflection Coefficient
     am,m=km                                      Update Predictor Coefficients

for k=1 to m-1

am,k=am-1,k+km⋅am-1,m-k  Update Predictor Coefficients

end

Em=Em-1⋅(1-km2) 	           Update the Prediction Error

end

Calculate Reflection Coefficient km​:
Use past coefficients and autocorrelation:

km=-rm+k=1m-1am-1,k⋅rm-kEm-1​

km: a "dummy variable" that measures the contribution of the new coefficient.

- Update Predictor Coefficients:

am,m=km

For k=1 to m-1:

am,k=am-1,k+km⋅am-1,m-k

- Update the Prediction Error:

Em=Em-1⋅1-km2

3. Repeat Until Desired Order p is Reached:
Continue the recursion until all predictor coefficients ap,k are computed for the p-th order model.

Result

- The final coefficients ap,k​ for k=1,2,…p are the Linear Prediction Coefficients (LPC) for the p-th order AR model.

- All lower-order LPCs (for 1 to p-1) are also computed during the process.

Advantages

- Efficient (computational cost is Op2 compared to Op3 for matrix inversion).

- Recursive computation avoids recalculating lower-order models from scratch.

The Levinson-Durbin algorithm is widely used in speech processing, signal prediction, and AR modeling due to its computational efficiency and numerical stability.

11. AR-filter design by modelling

Autoregressive (AR) filter design involves modeling a signal as the output of a linear system driven by white noise. The AR filter parameters are derived from the signal's statistical properties, such as autocorrelation, and are commonly used in signal processing, speech processing, and spectral analysis.

Key Steps for AR-Filter Design by Modeling

1. Model Assumption:
Assume the signal x[n] can be represented as an AR process:

xn= k=1pakxn-k+en

ak​: AR coefficients (filter parameters)

p: Order of the AR model

en: White noise (residual/error)

Yule-Walker eq.:  Ra=r
R: Toeplitz autocorrelation matrix. 
a: AR coefficient vector. 
r: Autocorrelation vector of lagged terms.

Yule-Walker eq.:  Ra=r
R: Toeplitz autocorrelation matrix. 
a: AR coefficient vector. 
r: Autocorrelation vector of lagged terms.

2. Autocorrelation-Based Estimation:
Compute the autocorrelation sequence rm of the signal, which captures its statistical dependence.

3. Solve Yule-Walker Equations:
Use the autocorrelation function to 
solve the Yule-Walker equations:

rm=-k=1pakrm-k,    m=1,2,…p

These equations establish a relationship between the AR coefficients and the autocorrelation values.

4. Levinson-Durbin Algorithm (Efficient Computation):

Use the Levinson-Durbin algorithm to recursively calculate the AR coefficients ak​ without directly solving the equations.

5. Filter Construction:

Construct the AR filter using the derived coefficients ak:

Hz=11+k=1pakz-k

Hz: the transfer function of the AR filter.

6. Stability Check:
Ensure the AR filter is stable by verifying that all poles lie within the unit circle in the z-plane.

7. Signal Modeling/Filtering:
Use the designed AR filter to:

- Generate synthetic signals by driving the filter with white noise.

- Analyze the signal's power spectrum (via the AR model).

- Predict or smooth the signal by applying the filter.

Applications

- Speech Processing: AR filters model vocal tract resonances.

- Spectral Estimation: AR modeling helps estimate the power spectrum of a signal.

- Signal Prediction: AR filters predict future values in time-series data.

By leveraging AR-filter design, signals can be effectively modeled, analyzed, and processed with high accuracy and computational efficiency.

12. The Lattice FIR filter

Core Concept:
The Lattice FIR filter uses a recursive structure to compute forward and backward prediction errors at each stage m, minimizing the residual error.

Equations for Writing:

- Forward Prediction Error: e(m)n=e(m-1)n-kme(m-1)n-1.

- Backward Prediction Error: e(m)n=e(m-1)n-1-kme(m-1)n.

- Final forward error gives the filter output: yn=e(m)[n].

This set of differences equations expresses the m-th order forward and backward prediction errors can be drawn as a Cascade of m Stages (one iteration of the Levinson-Durbin algorithm)

- Horizontal Arrows: Represent the forward and backward error path.

- Diagonal Arrows: Scale the forward and backward paths by the Reflection Coefficient km​.

- Delay Block (z-1): Indicate one-sample delay for the backward error.

Figure 1: Cascade of m stages

Figure 1: Cascade of m stages

At each stage, the forward error e(m-1)n is updated using the previous stage's forward and backward errors, scaled km​ (Reflection Coefficient).

- The cascade structure allows us to compute filter 
coefficients recursively, ensuring efficiency and stability.

In conclusion we can now state that instead of A(m)(z) being an ordinary tapped-delay-line FIR-filter it can be implemented as a cascade of m sections of the structure figure 1 – This is known as the Lattice Filter.  Note: For m=0, there’s no prediction, so the error is directly the input signal: e(0)n=: e(0)n=x[n]

FIR coefficients

The coefficients are calculated from the residual signals(error), and for order m becomes:

km=-n=0N=1(em-1n⋅e(m-1)n-1)n=0N=1em-1n2

NB: Should be numerical smaller than zero.

NOTE:

- 𝒊th filter coefficient is calculated based on residual signals from the i - 1st lattice stage!

- The forward and backward Residual variances are identical for order 𝒎.

- Compared to FIR implementation of LPC, we do not use the columns from the Levinson-Durbin algorithm, but the diagonal (km) ensuring values between −𝟏 and 𝟏

- This is good for fixed-point calculations (proof omitted)