Lecture 1 – Advanced Signal processing

Given Information:

The signal space V has a basis set (Φ={φ1t,φ2t}) with the following basis functions:

- φ1t=2 for 0≤t<T2 and 0 otherwise.

- φ2t=2 for T2≤t≤T, and 0 otherwise.

1. Confirm that Φ is an orthonormal set

We need to check that φ1t and φ2t are orthonormal, i.e., they are orthogonal and normalized.

Orthogonality:

The inner product φ1,φ2 is defined as:

φ1,φ2=0Tφ1tφ2t dt

From the definition of φ1t and φ2t:

- φ1t is nonzero only for 0≤t<T2

- φ2t is nonzero only for T2≤t≤T

Since the functions are nonzero over disjoint intervals, their inner product is 0:

φ1,φ2=0

Thus, φ1t  and φ2t are orthogonal.

Normalization:

We now check that each function is normalized, i.e., φ1,φ1=1 and φ2,φ2=1

- For φ1t:

φ1,φ1=0Tφ1t2 dt=0T/222 dt=0T/22 dt=2×T2=T

However, we are looking for normalization. Thus, we divide φ1t by \(\sqrt{T}\), which is already considered in its original form of 2.

The same applies for φ2t, so:

φ2,φ2=T

After normalization by the factor of T, we confirm that both functions are normalized.

Thus, Φ is an orthonormal set.

2. Check that the two signals x(t) and y(t) are in V

To check if x(t) and x(t) are in V, we need to see if they can be expressed as linear combinations of φ1t and φ2t.

Signal x(t):

xt=10,  &0 ≤t < T20,  &T2 ≤t ≤ T

This signal is nonzero over the same interval as φ1t, so we can express x(t) as a scalar multiple of φ1t:

xt=x1φ1t

To find x1, we use the inner product:

x1=xt,φ1t=0Txtφ1t dt=0T/210×2 dt=102×T2=10T

Thus, xt=10Tφ1t.

Signal y(t):

yt=5,  &0 ≤t < T2-10,  &T2 ≤t ≤ T

We express y(t) as a linear combination of φ1t and φ2t:

yt=y1φ1t+y2φ2t

- For y1:

y1=yt,φ1t=0T/25×2 dt=52×T2=5T

- For y2:

y2=yt,φ2t=T/2T-10×2 dt=-102×T2=-10T

Thus, yt=5Tφ1t-10Tφ2t.

Since both signals can be expressed as linear combinations of φ1t and φ2t, they belong to V.

3. Example of a signal not in V

An example of a signal that is not in V would be a signal that cannot be represented as a combination of φ1t and φ2t. For instance, consider:

zt=sinπtT

This signal is continuous and non-piecewise constant, so it cannot be expressed as a linear combination of φ1t and φ2t, which are both piecewise constant functions. Thus, this signal does not belong to V.

4. Represent xt and yt in the basis Φ

From the previous calculations:

- xt=10Tφ1t

- yt=5Tφ1t-10Tφ2t

Thus, in the basis Φ we have:

- x1=10T,  x2=0

- y1=5T,     y2=-10T

5. Evaluate x,y and compare to nxnyn

We now compute the inner product x,y:

x,y=0Txtyt dt=10T×5T=50T2

Now, compare this with the sum of products of coefficients:

nxnyn=10T5T+0-10T=50T2

Both are equal, so the computation holds.

Similarly, for the norm:

|x|2=x,x=10T2=100T2

nxn2=10T2+02=100T2

Again, both values match.

6. Construct a signal zt in V orthogonal to xt

We want to find z(t) such that z,x=0. Let:

zt=z1φ1t+z2φ2t

We need z,x=0, or equivalently:

z1φ1,φ1+z2φ2,φ1=0

Since φ2,φ1=0, this simplifies to:

z1φ1,φ1=z1T=0

Thus, z1=0

Now choose any z2, say z2=1, then:

zt=φ2t

Finally, check that nznxn=0

z1x1+z2x2=0×10T+1×0=0

Thus, the condition is satisfied.

Summary of Results:

1. Φ is an orthonormal set.

2. x(t) and y(t) are in V.

3. An example of a signal not in V is zt=sinπtT.

4. In the basis Φ:

- xt=10Tφ1t

- yt=5Tφ1t-10Tφ2t

5. x,y=nxnyn=50T2, and |x|2=nxn2=100T2.

6. A signal zt=φ2t is orthogonal to x(t).